| PRODUCT GROUP A : | Dell Server |  |  |
| --- | --- | --- | --- |
| PRODUCT SUB GROUP : | Dell PowerEdge Rack Servers Scalable, effective rack server solutions, deployable in any IT environment - providing high performance computing solutions |  |  |
| PRODUCT IMAGE : |  | PRODUCT NAME | PowerEdge R630 Rack Server |
|  |  | PRODUCT PRICE ( RM ) | Contact Us |
|  |  | PRODUCT WEIGHT ( g ) | Refer to Product Details |
|  |  | PRODUCT DIMENSION (cm ) | Refer to Product Details |
| PRODUCT BRIEF DESCRIPTION : |  |  |  |
| Concentrate your computing and capacity. Maximize data center efficiency with an ultra-dense virtualization or database engine that supports up to 24 flash SSDs in a 1U chassis. |  |  |  |
| PRODUCT DETAIL DESCRIPTION : |  |  |  |
| ### Processor - Intel® Xeon® E5 2600 v3 processors ### Operating System - Microsoft® Windows Server® 2008 R2 SP1, (includes Hyper-V®) Microsoft Windows Server 2012 Microsoft Windows Server 2012 R2 (includes Hyper-V) Novell® SUSE® Linux Enterprise Server Red Hat® Enterprise Linux ### Chipset - Intel C610 series chipset ### Memory1 - 24 DIMM slots, DDR4 memory Architecture: Up to 2133MT/s DDR4 DIMMs Memory Type: RDIMM, LRDIMM Memory Module Sockets: 24 Minimum RAM: 4GB (one module) Maximum RAM: Up to 768GB (24 DIMM slots): 4GB/8GB/16GB/32GB ### Embedded Hypervisor (Optional) - Microsoft Windows Server 2012, includes Hyper-V VMware® vSphere® ESXi™ Citrix® XenServer® ### Storage - HDD: SAS, SATA, nearline SAS SSD: SAS, SATA, PowerEdge Express Flash NVMe PCIe SSD 24 x 1.8” SSD – up to 23TB via 0.96TB hot-plug SATA SSD 10 x 2.5” – up to 18TB via 1.8TB hot-plug SAS HDD 8 x 2.5” – up to 14TB via 1.8TB hot-plug SAS HDD ### Drive Bays - Internal hard drive bay and hot-plug backplane: Up to 24 x 1.8” SATA SSD Up to 10 x 2.5” HDD: SAS, SATA, nearline SAS SSD: SAS, SATA, Up to 4 NVMe PCIe Up to 8 x 2.5” HDD: SAS, SATA, nearline SAS SSD: SAS, SATA ### Slots - 2 CPUs, 3 slots Slot 1: Half length, half height - PCIe 3.0 x16 (x16 connector) Slot 2: Half length, half height - PCIe 3.0 x8 (x16 connector) Slot 3: Half length, half height - PCIe 3.0 x16 (x16 connector) 2CPUs, 2 slots Slot 1: Half length, half height - PCIe 3.0 x16 (x16 connector) Slot 2: 3/4 length, full height - PCIe 3.0 x16 (x16 connector) 1CPU, 2 slots Slot 1: Half length, half height - PCIe 3.0 x8 (x16 connector) Slot 2: 3/4 length, full height - PCIe 3.0 x16 (x16 connector Dedicated RAID card slot ### RAID Controllers - Internal: PERC S130 (SW RAID), PERC H330 PERC H730 PERC H730P External: PERC H830 External HBAs (non-RAID): 12Gbps SAS HBA ### Network Controller - 4 x 1Gb, 2 x 1Gb + 2 x 10Gb, 4 x 10Gb ### Communications - Broadcom® 5719 quad-port 1Gb NIC Broadcom 5720 dual-port 1Gb NIC Broadcom 57810 dual-port 10Gb DA/SFP+ CNA Broadcom 57810 dual-port 10Gb Base-T network adapter Intel® Ethernet I350 dual-port 1Gb server adapter Intel Ethernet I350 quad-port 1Gb server adapter Intel Ethernet X540 dual-port 10GBASE-T server adapter Mellanox® ConnectX®-3 dual-port 10Gb Direct Attach/SFP+ server network adapter Mellanox ConnectX-3 dual-port 40Gb Direct Attach/QSFP server network adapter Emulex® LPE 12000, single-port 8Gb Fibre Channel HBA Emulex LPE 12002, dual-port 8Gb Fibre Channel HBA Emulex LPe16000B, single-port 16Gb Fibre Channel HBA Emulex LPe16002B, dual-port 16Gb Fibre Channel HBA Emulex OneConnect OCe14102-U1-D 2-port PCIe 10GbE CNA QLogic® 2560, single-port 8Gb Optical Fibre Channel HBA QLogic 2562, dual-port 8Gb Optical Fibre Channel HBA Qlogic 2660, single-port 16GB, Fibre Channel HBA, full height Qlogic 2662, dual-port 16GB, Fibre Channel HBA, full height ### Power - 1100W AC, 86 mm (Platinum) 1100W DC, 86 mm 750W AC, 86 mm (Platinum) 750W AC, 86 mm (Titanium) 495W AC, 86 mm (Platinum) ### Availability - ECC memory Hot-plug hard drives Hot-plug redundant cooling Hot-plug redundant power Internal Dual SD Module Single Device Data Correction (SDDC) Spare Rank Support for high availability clustering and virtualization Proactive systems management alerts iDRAC8 with Lifecycle Controller ### Chassis - The 24 x 1.8” drive and 10 x 2.5” drive configurations have the following dimensions: Form Factor: 1U H: 4.28 cm (1.68 in.) x W: 48.23 cm (18.98 in.) x D: 75.51 cm (29.72 in.) The 8 x 2.5” drive configuration has the following dimensions: Form Factor: 1U H: 4.28 cm (1.68 in.) W: 48.23 cm (18.98 in.) D: 70.05 cm (27.57 in.) ### Management - Systems management: IPMI 2.0 compliant; Dell OpenManage Essentials; Dell OpenManage Mobile; Dell OpenManage Power Center Dell OpenManage Integrations: Dell OpenManage Integration Suite for Microsoft System Center Dell OpenManage Integration for VMware® vCenter Dell OpenManage Connections: HP Operations Manager, IBM Tivoli® Netcool® and CA Network and Systems Management Dell OpenManage for Oracle Database Manager Remote management iDRAC8 with Lifecycle Controller, iDRAC8 Express (default) iDRAC8 Enterprise (upgrade) 8GB vFlash media (upgrade)16GB vFlash media (upgrade) iDRAC Quick Sync |  |  |  |